# CSC 450- Campus Network

# Project (2)- Brainstorm Phase

# Dr. Majd Tahat

Big picture 
You will design, implement, and configure a multi-building campus network for Louisiana Tech using Cisco Packet Tracer. Pick your buildings from the campus map, assign roles, decide VLAN IDs and addressing, and prove a working baseline with DHCP, DNS+HTTP, inter-VLAN connectivity, NAT, Wi-Fi (single SSID), and basic ACL protections.

### Host-Capacity (you must size masks to meet or exceed)

| Role (your chosen VLAN ID) | Minimum usable hosts the subnet must support |
| --- | --- |
| Admin | ≥ 254 (at least /24 capacity) |
| Faculty | ≥ 254 |
| Student | ≥ 254 |
| Residence | ≥ 254 |
| Research | ≥ 254 |
| Servers | ≥ 14 (at least /28 capacity) |
| Transit (Core↔Edge) | 2 (i.e., /30 point-to-point) |

You may choose larger subnets than these minimums. All subnets must be private IPv4 and non-overlapping.

### Student-Choice (Freedom)

| Area | What you decide | Constraints you must satisfy |
| --- | --- | --- |
| Building selection | Choose ≥ 7 buildings from the university map (label them in your diagram) | Use real names from the uploaded map |
| Role placement across buildings | Which buildings host Admin, Faculty, Student, Residence, Research, Servers | Every role appears ≥ 1×; exactly one building hosts Servers (DNS/HTTP). Include ≥ 2 buildings with Admin or Faculty endpoints. |
| VLAN IDs | Choose the VLAN ID for each role | Map them clearly in your tables/diagram |
| Addressing plan | Private block(s), masks per role, DHCP pool bounds, transit /30, SRV1 IP | Must meet host-capacity (see table below), be non-overlapping, and be summarized in Part 1 |
| Inter-VLAN routing placement | L3 at Core SVIs or Router-on-a-Stick or Distribution SVIs | One clear default route toward the edge NAT device |
| NAT device placement | Which router at the edge performs NAT/PAT | Must be at the campus edge; outbound NAT for all inside subnets |
| Distribution / Access design | One or two Distribution nodes; one Access switch per chosen building; your trunk map | Use tight allow-lists on trunks |
| Wi-Fi footprint | Which chosen buildings have an AP | Minimum 2 Wi-Fi buildings; AP switchports are access to the Student VLAN |
| Diagram style | How your high-level topology looks | Must show buildings, access/distribution/core/edge, services, and Wi-Fi sites |

### Locked (Fixed Defaults)

| Area | Fixed requirement (implement exactly) |
| --- | --- |
| Gateway convention | The SVI gateway in every VLAN is .1 |
| DHCP host | CORE-SW1 serves DHCP for all VLANs (pools you define to match your masks) |
| DNS/HTTP | Zone latech.local with only www.latech.local → SRV1 (use the SRV1 IP you chose). Single index page. SRV1 resides in the Servers VLAN. |
| NAT policy | Outbound NAT/PAT for all inside subnets at the edge; inbound publish HTTP (TCP/80) only to SRV1 |
| ACL placement & scope | Enforce at least: (1) Student & Residence → DENY SSH/Telnet/SNMP to infrastructure (apply at source SVIs), (2) Allow DNS(53)/HTTP(80) to SRV1, (3) Edge inbound: permit TCP/80 to SRV1 then deny other unsolicited inbound |
| Wi-Fi SSID | One SSID LT-Student bridged to the VLAN you designate as Student (whatever ID you chose) |
| AP port mode | AP switchports are access to the Student VLAN (not trunks) |
| Trunk policy | Trunks must use tight allow-lists (only VLANs actually needed beyond that link) |
| Deliverables | Part 1: Design PDF/slides + skeleton .pkt. Part 2: working .pkt + concise evidence PDF (filenames below). |