Louisiana Tech University

ELEN 479 Automatic Control Systems Lab

Lab 5: System Stability

Task 1

Let’s create a Routh Array in the MATLAB. Consider a system with the characteristic equation:

qs=s4+10s3+32s2+37s+20

Determine its stability.

Step 1: All the coefficients are positive and there are no missing coefficients.

Step 2: Routh array

| s4 | 1 | 32 | 20 |
| --- | --- | --- | --- |
| s3 | 10 | 37 | 0 |
| s2 | 10×32-1×3710=28.3 | 10×20-1×010=20 | 0 |
| s1 | 28.3×37-10×2028.3=29.9 | 28.3×0-10×028.3=0 | 0 |
| s0 | 29.9×20-28.3×029.9=20 | 0 | 0 |

The elements in the first column are all positive -> system is stable.

Task 2

Figure 1 shows the block diagram of the torsion control plant.

Task 2.1: Starting with the block diagram of the torsional control plant, reduce it to an equivalent closed-loop transfer function. Assume Km=τm=1.

Reference Solution: Kps2+1+Kds+Kp

For Km=τm=1.

1s+11+Kds+1=1s+1s+1+Kds+1=1s+1+Kd

Kp×1s+1+Kd×1s=Kps2+1+Kds

Kps2+1+Kds1+Kps2+1+Kds=Kps2+1+Kdss2+1+Kds+Kps2+1+Kds=Kps2+1+Kds+Kp

Task 2.2: Apply the Routh-Hurwitz criterion to the denominator of this function to determine the conditions for stability.

s2+1+Kds+Kp

| s2 | 1 | Kp |
| --- | --- | --- |
| s1 | 1+Kd | 0 |
| s0 | (1+Kd)Kp-1(0)1+Kd=Kp | 0 |

The system will be stable for Kd>-1 and Kp>0

Task 2.3: Implement Figure 1 in the Simulink. Set the input signal as a step function. Choose values of Kp and Kd to make the system output stable.

Task 2.4: Implement Figure 1 in the Simulink. Set the input signal as a step function. Choose values of Kp and Kd to make the system output unstable.